Mad heads

TECHNICAL RIDER

Musicans:

- Vadym Krasnooky: LeadVocal + ElectricGuitar

- Maksim Krasnooky: BackVocal + BassGuitar

- Volodymyr Zumchenko:  Drums

- Stanislav Semilietov: ElectricGuitar+ BackVocal

- Vadyn Nikitan Trumpet + BackVocal

- Valeriy Chesnokov Trombone + BackVocal

- Soundengineer

Contacts:

Producer, G.M.:

Oksana Mikhailenko  | +380 50 358 72 73 | zloy.director@gmail.com

Manager

Inna Sirosh | +380 50 038 03 88 | higearua@gmail.com

*The ARTIST shall be given a fair sound check with complete access to stage and sound for no less then three hours and this must occur before doors opens. On festivals the ARTIST will adapt to festival schedule.

*Before ARTISTS performance there must be 20 minutes changeover. It is very much appreciated if there is a good planning so there is not so much “dead time” for the crowd.

PA

Sound reinforcement shall be three or four way active system (L'Acoustic, Turbosound, Meyer). The PA must be able to deliver an undistorted sound of 110 dB(A) full range 20Hz to 20kHz at F.O.H and must be sufficient considering the venue and amount of people.

Full access to amplifiers and processors is required.

Front of house

DIGITAL CONSOLE:

AVID/DIGIDESIGN,  ALLEN&HEATH, YAMAHA, MIDAS

or

ANALOG CONSOLE:

Allen&Heath, Midas, Soundcraft,  (min 24 channel)

9 AUX min (6 AUX-PRE monitoring, 3 AUX-POST FX)

Depending on the concert can be require separate monitor mixer onstage

Effect rack (if Analog console)

2    x  Master Equalizer (1/3 octave): dbx, KLARK, BSS

4    x  Monitor Eq(1/3 octave).

10  x Compressors - dbx 160,166, 266, Drawmer, BSS

8    x Gates - dbx 166,Drawmer, BSS.

2    x Reverb / multi-effect (Lexicon PCM 70/80/90, TC M2000 or similar)

1    x Delay (Lexicon MX 400, TC D two or similar)

MONITORS

6 x wedge monirtors, should all be same type and good quality. Recommended 15” + 2” in pairs for each monitor   mix (minimum 500W RMS capacity).

1 x Drumfill. An additional sub-bass cabinet is required for the drumfill

2 x Fullstack SideFills (for open-airs)

Backline

Drum set

Tama Starclassic Performer BB bubinga/birch or Tama Starclassic maple on RISER

3

4

5

6

2

b

d

f

g

a

c

1

e

1. 22x18 or (24x16)KickDrum

2. 10x8                      RackTom

3. 12x9		      RackTom

4. 16x14	      FloorTom

5. 14x5 		      SnareDrum KennyAronoff k145 with diecast hoop

6. 12x5.5	      Snare metallwork

cymbals

a. right 22 	Ziljian  A custom

b. china 20	Ziljian  avedis

c. crash 19 	Ziljian  A custom

d. crash 17 	Ziljian A custom

e. slplash 10 	Ziljian A custom

f. hihat 1 	Ziljian K z1 dynobeat

g. hihat 2 	Ziljian avedis quksbeat

drum heads

Remo Emperor Clear for toms

Remo Ambassador Coated for snares

Remo Powerstrok3 for kick

hardwear

1 x HiHat stand tama ironcobra veloglide hh805/tama roadpro hh75wn

1 x mxa73 closed HiHat attachment

2 x roadpro snare stand hs70wn

5 x hc73bwn cimball stands

1 x multyclamp mc62

Pedal – Perl double elimination with blue eccentric

DrumThrone tama ht741 ergo-rider with backrest

1 x Cooler(for fresh air)!!!

1 x 4-ch mixer

2 x DI boxes

1 x stand for Laptop and 4-ch mixer(for playback)

GUITAR SET

Guitar 1 SL:

1 x VOX AC30CC

Guitar 2 SR

1st choice:

1 x VOX AC30CC

2nd choice:

Marshall JCM900 or Mesa Stiletto head + 4x12 cab.

2 x SM57 microphones

2 x Mic stands for guitar mics

3 x Guitar stands

All necessary Power and Speaker cables

BASS SET

1 x MESA BOOGIE, AMPEG, HARTKE (400W minimum)

1 x DI Box

1 x Guitar Stand

All necessary Power and Speaker cables

BRASS SET

2 x Vocal microphone stand with folding tripod base boomarm

2 x SM58 Vocal microphone

1 x Riser for Brass Section

Vocals

3 x Mic stands

2 x SM58 Vocal microphones (back vocals)

1 x Wireless Vocal microphone for lead vocal

MISC:

6 x 10 Meter Jack to Jack cable
4 x wireless sinnheiser ew 572 G3 or Shure ULX S14 instrument set (bodypack transmitter for guitars + cable + RF ressiver)

2 x wireless Shure ULX S14 mic set (bodypack transmitter for mic + cable + RF ressiver)

UP STAGE

DOWNSTAGE

guitar

combo

DRUMS

Drum fill MIX #5

Side fill

left MIX #8(9)

Side fill

right mix #8

wedge

mix #4

wedge

mix #4

mic

wedge

mix #1

wedge

mix #1

mic

wedge

mix #3

wedge

mix #3

mic

wedge

mix #7

wedge

mix #7

mic

MAD HEADS STAGE PLOT

Front of stage

stage right

stage LEFT

Bass combo

BASS

LEAD

Guitar

guitar combo

Laptop

2 DI

Brass Section

Monitor list

| Mix # |  | Description | Type | Position |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Mono | Vadym | 2 x Wedge | DownStageCentre |  |
| 2 | Mono | Vadym | IEM RF |  |  |
| 3 | Mono | Guitar | 2 x Wedge | DownStageRight |  |
| 4 | Mono | Bass | 2 x Wedge | DownStageLeft |  |
| 5 | Mono | Drum | Drumfill + Sub | UpStageCentre |  |
| 6 | Mono/Stereo | Drumxlr IEM | XLR(IEM) miniJack | UpStageCentre |  |
| 7 | Mono | Brass Section | 2 x Wedge | UpStageRight |  |
| 8 | Mono | Side-fillRight | FullStack | StageRight |  |
| 8(9) | Mono | Side-fillLeft | FullStack | StageLeft |  |

Input List

| Chann.# | Instrument | mic | inserts |  |
| --- | --- | --- | --- | --- |
| 1 | Kick | BETA52A | Gate+comp |  |
| 2 | Kick | BETA91A | Gate+comp |  |
| 3 | Snaretop | SM57/Beta56A | Compressor |  |
| 4 | Snarebottom | BETA98 | Gate |  |
| 5 | Secondsnaretop | BETA87 |  |  |
| 6 | Hi-hat | KSM137/SM81 |  |  |
| 7 | Tom 1 | BETA98A | Gate+comp |  |
| 8 | Tom 2 | BETA98A | Gate+comp |  |
| 9 | Flortom | BETA98AMP | Gate+comp |  |
| 10 | Ride | BETA181/KSM141 |  |  |
| 11 | OH L | KSM141/SM81/AKG C451 |  |  |
| 12 | OH R | KSM141/SM81/AKG C451 |  |  |
| 13 | Bass | DI-BOX | Compressor |  |
| 14 | Guitar VOX/Fender | SM57 | Compressor |  |
| 15 | GuitarVox/Marshall | SM57 | Compressor |  |
| 16 | Trombone | Beta98h/c | Compressor |  |
| 17 | Trumpet | Beta98h/c | Compressor |  |
| 18 | Backvoc. Brass1 | SM58 |  |  |
| 19 | Backvoc. Brass 2 | SM58 |  |  |
| 20 | Leadvocal | SM58 | Compressor |  |
| 21 | Backvoc. bass | SM58 | Compressor |  |
| 22 23 | Playback-L Playback-R |  |  |  |
| 24... | Talkback |  |  |  |
| FX returns |  |  |  |  |
| 1 | Drumroom – L |  |  |  |
| 2 | Drumroom – R |  |  |  |
| 3 | PitchShift/Octaver -12 |  |  |  |
| 4 | PitchShift/Octaver -12 |  |  |  |
| 5 | Hall – L |  |  |  |
| 6 | Hall – R |  |  |  |
| 7 | Tap – delaytriplet |  |  |  |
| 8 | Tap – delay ¼ |  |  |  |